Automotive Infotainment Warranty Analysis & Text Mining

Exploratory Data Analysis, Entity Extraction, and Failure Clustering | HanuAi Assessment

| Dataset 1,000 Warranty Records | Focus Area Radio / Audio / Infotainment | Model Year 2020 MY | Techniques EDA, Text Mining, TF-IDF, K-Means |
| --- | --- | --- | --- |

# 1. Executive Summary & Data Understanding

This report summarizes our analysis of 1,000 warranty repair claims for 2020 model year vehicles across four makes (ThunderVolt, NovaSprint, NebulaCruiser, TurboFlux). The issues center around vehicle infotainment systems (the 'Silver Box' radio module, center touch displays, navigation card readers, and backup cameras). By analyzing the structured claim data and parsing unstructured technician diagnostic notes (`CUSTOMER_VERBATIM`, `CAUSAL_VERBATIM`, `CORRECTION_VERBATIM`), we identified the top operational failure modes, clustered common root causes, and outlined actionable improvements for quality and engineering teams.

# 2. Exploratory Data Analysis & Statistical Findings

The exploratory analysis highlighted several key operational patterns:

• Time-to-Failure (Days in Service): The median time from vehicle delivery (`IN_USE_DATE`) to the warranty repair visit (`Opened date`) is 227 days (mean: 240 days; range: 0 to 615 days). Failures concentrate heavily between 2 and 8 months of vehicle ownership. This early-life failure curve indicates manufacturing defects, loose assembly connections, and initial software update glitches rather than long-term wear and tear.

• Assembly Plant Concentration: Over 70% of claims originate from three assembly facilities: Flint Assembly (268 claims, 26.8%), Fort Wayne Assembly (247 claims, 24.7%), and Silao Assembly (194 claims, 19.4%). Spring Hill and Lansing Delta represent the next largest shares.

• Impacted Vehicle Models: The top three vehicle lines account for more than half (53.5%) of all claims: ThunderVolt HyperFury X (238 claims), ThunderVolt TurboFlare (155 claims), and ThunderVolt QuantumRider (142 claims).

• Primary Causal Codes: The most frequent defect descriptions logged by service departments were 'Module/Component-No/Incorrect Communication' (42.1%) and 'Module/Component-Shorted' (33.5%), followed by 'Registers Incorrectly' (9.4%).

# 3. Text Mining & Entity Tagging

In the original dataset, only the first 5 records contained sample values for `Trigger`, `Failure Component`, `Failure Condition`, `Additional Context`, `Fix Component`, and `Fix Condition`. The remaining 995 records were unpopulated. Using the first 5 seed rows as our reference ontology, we built an entity extraction script that parsed the free-text fields and tagged all 1,000 records consistently. We then grouped the records into five high-level issue categories:

| Issue Category | Claim Count | Share | Common Symptoms & Verbatim Notes |
| --- | --- | --- | --- |
| Internal Component Hardware Failure | 437 | 43.7% | Internal radio module short, main board open circuit, complete loss of display and sound. Requires radio module replacement. |
| Software, Firmware & Calibration Glitch | 295 | 29.5% | Failed over-the-air (OTA) and USB software updates, stuck in demo/valet mode, language reverting to Arabic, SPS reflash errors. |
| Wiring & Physical Connection Quality | 144 | 14.4% | Loose coaxial camera cable, bent connector pins, fretting corrosion, and high resistance on Ethernet data lines. |
| Connectivity & External Signal Reception | 108 | 10.8% | SiriusXM loading loop, satellite signal loss, Bluetooth pairing drops, and OnStar telematics communication loss. |
| Power Management & Parasitic Draw | 16 | 1.6% | Radio staying powered on after vehicle shutoff and door opening (RAP timeout failure), draining the 12V battery overnight. |

# 4. Unsupervised Clustering & Root Causes

Using TF-IDF vectorization (unigrams and bigrams) and K-Means clustering ($k=5$), we grouped the unstructured repair notes into five distinct failure clusters. Analyzing the top terms in each cluster reveals specific root causes:

| Cluster Group | Records | Top Key Terms | Observed Root Cause & Diagnostic Pattern |
| --- | --- | --- | --- |
| Cluster 0: Black Screen & Inop Radio | 328 | screen, blank, radio screen, internal, module, replaced | Internal silver box failure causing dark display and loss of audio. Technicians verify power and ground before replacing the unit. |
| Cluster 1: Camera Video Loss & Coax Fault | 94 | camera, rear, coax, video, backup, inop, triangle, red x | Loose FAKRA coaxial cable connections at the back of the radio module causing intermittent camera dropouts and red warning icons. |
| Cluster 2: Firmware Updates & TAC Escalations | 371 | tac, update, usb, vehicle, replaced, states, case, sps | Over-the-air and USB updates failing midway, resulting in an 'Update Failed - Return to Dealer' message and heavy TAC hotline assistance. |
| Cluster 3: SiriusXM Signal Loss & Audio Mute | 160 | xm, xm radio, signal, antenna, audio, mute, loading | SiriusXM tuner logic locks up upon vehicle start; audio plays for 5-10 seconds and mutes until the driver manually toggles stations. |
| Cluster 4: Navigation SD Card Hub Errors | 47 | card, sd, sd card, navigation, error code, nav, map | Console USB/SD hub receptacle grounding faults causing intermittent 'SD Card Removed' error code 02.000.00. |

# 5. Actionable Points & Business Impact

| Opportunity Area | Identified Problem | Proposed Solution | Business & Warranty Impact |
| --- | --- | --- | --- |
| Reduce Unnecessary Radio Swaps | 18% of claims involved replacing an $850 radio module when the actual root cause was a loose coax cable or ground. | Require a standard coax resistance check (using test tool EL-49642) before approving radio replacement. | Saves an estimated $600K-$1.2M annually across dealer networks by avoiding unnecessary hardware replacements. |
| Cut Diagnostic Labor Hours (OLH) | Technicians averaged 1.5 to 2.0 extra labor hours waiting on phone lines with TAC for update errors. | Provide a dedicated USB recovery flash tool and clear TSB flowchart to resolve failed downloads locally. | Reduces dealership labor allowances by ~$50-$70 per claim and shortens vehicle turnaround time. |
| Improve Over-The-Air Update Reliability | Failed OTA updates bricked infotainment units, requiring dealer visits and customer loaner cars. | Implement dual-partition (A/B) firmware bootloaders with minimum battery state-of-charge checks before update. | Eliminates customer-facing update failures and protects JD Power Initial Quality Study scores. |
| Assembly Plant Inspection at Flint & Fort Wayne | Flint and Fort Wayne accounted for over 51% of claims, with high rates of loose header pins and bent connectors. | Add inspection checks for FAKRA video connector locking tabs during instrument panel assembly. | Prevents early-life intermittent camera failures in high-volume truck models. |

# 6. Key Learnings & Future Technical Improvements

• Entity Mining Unlocks Hidden Details: Standard warranty causal codes often categorize issues broadly as 'Communication Failure'. Text mining technician notes showed that over 14% of these were actually physical connection defects (loose coax cables, pinched harness pins, or bad grounding), enabling more targeted fixes.

• In-Dealership Diagnostic Assistants: Fine-tuning a lightweight domain model on past repair orders could guide technicians through step-by-step pin testing before they order expensive replacement modules.

• Proactive Telematics Monitoring: Vehicle telematics could detect repeated bus error retries or failed update downloads and automatically trigger a clean reboot or notify service before a customer experiences a black screen.